Race Timer
BLE Integration

iOS App ↔ Mainboard Communication Protocol

## System Architecture

• iOS App sends commands → Mainboard 
• Mainboard sends events (notifications) → iOS App
• iOS App can read the current race status at any time
• Mainboard handles all timing with hardware TIMER
• 4 race buttons + 1 auxiliary button (start/stop)
• 4-digit seven-segment display (MAX7221 via SPI)
• 5 button LEDs  + 1 status LED

## Key Design Principles

1. Mainboard is the source of truth for elapsed time.
2. All times are in milliseconds (uint32_t, little-endian).
3. The race continues even if BLE disconnects mid-race (error state).
4. Button 5 allows standalone operation without the iOS app.
5. The iOS app should subscribe to notifications before starting a race.

# 1. BLE Connection Details

| Parameter | Value |
| --- | --- |
| Device Name | RaceTimer |
| Appearance | 768 (0x0300) |
| Advertising Type | Connectable, Undirected |
| Max Connections | 5 |
| Max Paired Devices | 5 |
| BLE Features | SMP (Secure Simple Pairing) |
| Advertising Data | Flags + 128-bit Service UUID |
| Scan Response | Complete Local Name ("RaceTimer") |

# 2. GATT Service & Characteristics

## Service UUID

00001000-7261-6365-7469-6D6572303031

## Characteristics

| Characteristic | UUID | Properties | Direction |
| --- | --- | --- | --- |
| Command | 00001001-7261-6365-7469-6D6572303031 | Write / Write Without Response | iOS → Mainboard |
| Event | 00001002-7261-6365-7469-6D6572303031 | Notify | Mainboard → iOS |
| Status | 00001003-7261-6365-7469-6D6572303031 | Read | Mainboard → iOS |

## Characteristic Details

### Command Characteristic (Write)

The iOS app writes to this characteristic to send commands to the mainboard. Supports both Write and Write Without Response. Each write must contain a 4-byte command packet.

### Event Characteristic (Notify)

The mainboard sends real-time race events via BLE notifications on this characteristic. The iOS app MUST enable notifications (subscribe) on this characteristic before sending any race commands. Each notification contains an 11-byte event packet.

⚠ IMPORTANT: Enable notifications on the Event characteristic immediately after connecting, before sending any commands.

### Status Characteristic (Read)

A readable characteristic that returns the current race state as a single byte (uint8). The iOS app can read this at any time to check the mainboard's current state.

# 3. Command Protocol (iOS → Mainboard)

## Command Packet Structure (4 bytes)

| Byte | Field | Type | Description |
| --- | --- | --- | --- |
| 0 | cmd_type | uint8 | Command ID (see table below) |
| 1 | mode | uint8 | Race mode (for CMD_SET_MODE only, otherwise 0x00) |
| 2 | reserved | uint8 | Reserved — set to 0x00 |
| 3 | reserved | uint8 | Reserved — set to 0x00 |

## Command IDs

| Command | ID (Hex) | Description | Valid States | Response Event |
| --- | --- | --- | --- | --- |
| CMD_START_RACE | 0x01 | Start the race timer | CONFIGURED, ARMED | EVT_START_ACK |
| CMD_NEW_RACE | 0x02 | Reset and prepare for a new race | Any | (state changes to IDLE) |
| CMD_RESET | 0x03 | Full reset (same as NEW_RACE) | Any | (state changes to IDLE) |
| CMD_PING | 0x04 | Ping — check mainboard is alive | Any | EVT_PONG |
| CMD_GET_STATUS | 0x05 | Request current race status | Any | EVT_STATUS |
| CMD_SET_MODE | 0x06 | Set the race mode (byte 1 = mode) | IDLE, CONFIGURED | (state → CONFIGURED) |

## Command Examples (Raw Bytes)

Set Mode to 4-Contestant (Mode 1):

[0x06, 0x01, 0x00, 0x00]   // CMD_SET_MODE, mode=0x01

OR

Set Mode to 1-Contestant / 4-Segment (Mode 2):

[0x06, 0x02, 0x00, 0x00]   // CMD_SET_MODE, mode=0x02

-----------------------------------------------------------------------------------------------------------------------

Start the Race:

[0x01, 0x00, 0x00, 0x00]   // CMD_START_RACE

Ping:

[0x04, 0x00, 0x00, 0x00]   // CMD_PING

Get Status:

[0x05, 0x00, 0x00, 0x00]   // CMD_GET_STATUS

New Race / Reset:

[0x02, 0x00, 0x00, 0x00]   // CMD_NEW_RACE

# 4. Event Protocol (Mainboard → iOS)

## Event Packet Structure (11 bytes)

| Byte(s) | Field | Type | Description |
| --- | --- | --- | --- |
| 0 | event_type | uint8 | Event ID (see table below) |
| 1 | mode | uint8 | Current race mode (0x01 or 0x02) |
| 2 | button_index | uint8 | Button number (1–4), or 0 if N/A |
| 3–6 | elapsed_time_ms | uint32 (LE) | Elapsed time in milliseconds (little-endian) |
| 7–10 | race_id | uint32 (LE) | Unique race identifier (little-endian) |

Note: All multi-byte integers are in little-endian byte order (standard for BLE / ARM).

## Event IDs

| Event | ID (Hex) | Description | Key Fields |
| --- | --- | --- | --- |
| EVT_START_ACK | 0x10 | Race has started — timer is now running | elapsed_time_ms = 0, race_id = new ID |
| EVT_STOP_EVENT | 0x11 | A button was pressed during the race | button_index = 1–4, elapsed_time_ms = time of press |
| EVT_RACE_COMPLETE | 0x12 | Race is fully complete | button_index = winner (Mode 1) or 0, elapsed_time_ms = final time |
| EVT_STATUS | 0x13 | Current status response (to CMD_GET_STATUS) | elapsed_time_ms = current elapsed time |
| EVT_PONG | 0x14 | Ping response (mainboard is alive) | elapsed_time_ms = 0 |
| EVT_ERROR | 0x1F | Error occurred (invalid command, etc.) | all fields may be 0 |

## Parsing Example (11-byte notification)

When you receive a notification on the Event characteristic, parse the 11 bytes:

App Sends [0x06, 0x01, 0x00, 0x00]   // CMD_SET_MODE, mode=0x01

Race Starts App Sends [0x01, 0x00, 0x00, 0x00]   // CMD_START_RACE

When first time button pressed after race start
Example received bytes: [0x11, 0x01, 0x02, 0xE8, 0x03, 0x00, 0x00, 0xAB, 0xCD, 0x00, 0x00]

Parsed:
  event_type      = 0x11 → EVT_STOP_EVENT
  mode             = 0x01 → Mode 1 (4 contestants)
  button_index     = 0x02 → Button 2
  elapsed_time_ms  = 0x000003E8 → 1000 ms (1.000 seconds)
  race_id          = 0x0000CDAB → 52651

When second time button pressed after race start

Example received bytes: [0x11, 0x01, 0x01, 0x88, 0x13, 0x00, 0x00, 0xAB, 0xCD, 0x00, 0x00]

Parsed:
  event_type      = 0x11 → EVT_STOP_EVENT
  mode             = 0x01 → Mode 1 (4 contestants)
  button_index     = 0x02 → Button 1
  elapsed_time_ms  = 0x00001388 → 5000 ms (5.000 seconds)
  race_id          = 0x0000CDAB → 52651

Race completed with winner

Example received bytes: [0x12, 0x01, 0x02, 0xE8, 0x03, 0x00, 0x00, 0xAB, 0xCD, 0x00, 0x00]

Parsed:
  event_type      = 0x11 → EVT_RACE_COMPLETE
  mode             = 0x01 → Mode 1 (4 contestants)
  button_index     = 0x02 → Button 2
  elapsed_time_ms  = 0x000003E8 → 1000 ms (1.000 seconds)
  race_id          = 0x0000CDAB → 52651

[0x02, 0x00, 0x00, 0x00]   // CMD_NEW_RACE

App Sends [0x06, 0x01, 0x00, 0x00]   // CMD_SET_MODE, mode=0x01

Race Starts App Sends [0x01, 0x00, 0x00, 0x00]   // CMD_START_RACE

# 5. Race States

## State Machine

The mainboard follows a strict state machine. The Status characteristic reflects the current state value.

| State | Value | Description | LED Indicators |
| --- | --- | --- | --- |
| DISCONNECTED | 0 | No BLE connection | Status LED: fast blink (200ms on / 800ms off), Button LEDs: OFF |
| IDLE | 1 | BLE connected, no race configured | Status LED: solid ON, Button LEDs: OFF, Display: "----" |
| CONFIGURED | 2 | Mode set, ready to start | Status LED: solid ON, Display: "00:00" |
| ARMED | 3 | Ready to start race | Status LED: solid ON, Button LEDs: ON for active lanes |
| RUNNING | 4 | Race in progress, timer counting | Status LED: solid ON, Display: running elapsed time |
| FINISHED | 5 | Race complete, results available | Status LED: solid ON, Winner button LED: ON |
| ERROR | 6 | Error (e.g. BLE lost during race) | Status LED: rapid blink (100ms/100ms), Display: "----" |

## State Transitions

DISCONNECTED → IDLE                     (BLE connects)
IDLE → CONFIGURED                       (CMD_SET_MODE received)
CONFIGURED → RUNNING                    (CMD_START_RACE received)
ARMED → RUNNING                         (CMD_START_RACE received)
RUNNING → FINISHED                      (all buttons pressed or manual stop)
RUNNING → ERROR                         (BLE disconnects during race)
ERROR → RUNNING                         (BLE reconnects, timer still running)
ERROR → IDLE                            (BLE reconnects, timer stopped)
FINISHED → IDLE                         (CMD_NEW_RACE / CMD_RESET)
Any State → IDLE                        (CMD_RESET)
Any State → DISCONNECTED                (BLE disconnects, except during race)

# 6. Race Modes

## Mode 1: 4-Contestant Race (0x01)

• 4 independent contestants, each assigned one button (1–4).
• All buttons are active. The first button pressed is the winner.
• After the winner is declared, the remaining buttons can still be pressed
  to record 2nd, 3rd, and 4th place times.
• A 30-second grace period runs after the first button press:
  – If all 4 buttons are pressed within 30 seconds, race ends immediately.
  – If the grace period expires, race auto-finishes with available results.
• Button 5 (auxiliary) can manually stop the race at any time.
• Each EVT_STOP_EVENT includes the button_index and elapsed_time_ms.
• The final EVT_RACE_COMPLETE includes the winner's button and time.

## Mode 2: 1-Contestant / 4-Segment Race (0x02)

• 1 contestant completes 4 sequential segments.
• Buttons 1–4 are pressed in order as the contestant passes each checkpoint.
• Each button press records a segment/split time.
• The race completes when all 4 buttons have been pressed.
• EVT_STOP_EVENT is sent for each segment with elapsed_time_ms from race start.
• EVT_RACE_COMPLETE is sent when all 4 segments are done, with total elapsed time.
• Button 5 can manually stop/end the race at any time.

| Mode | Value | Contestants | Buttons Used | Completion Condition |
| --- | --- | --- | --- | --- |
| 4-Contestant | 0x01 | 4 | Buttons 1–4 (independent) | All 4 pressed, or 30s grace after 1st |
| 1-Contestant | 0x02 | 1 | Buttons 1–4 (sequential) | All 4 segments completed |

# 7. Typical Flow Examples

## Example 1: Mode 1 — 4-Contestant Race

Step-by-step sequence for a typical 4-contestant race:

| Step | Action | Direction | Bytes / Event |
| --- | --- | --- | --- |
| 1 | iOS scans & connects to "RaceTimer" | — | State → IDLE |
| 2 | iOS enables notifications on Event char | iOS → FW | Write to CCCD: [0x01, 0x00] |
| 3 | iOS sets Mode 1 | iOS → FW | Write: [0x06, 0x01, 0x00, 0x00] |
| 4 | (State changes to CONFIGURED) | — | Status char = 0x02 |
| 5 | iOS sends Start Race | iOS → FW | Write: [0x01, 0x00, 0x00, 0x00] |
| 6 | FW acknowledges start | FW → iOS | Notify: EVT_START_ACK (0x10) |
| 7 | Contestant 3 presses button first | FW → iOS | Notify: EVT_STOP_EVENT, btn=3, time=2345ms |
| 8 | Contestant 1 presses button | FW → iOS | Notify: EVT_STOP_EVENT, btn=1, time=2890ms |
| 9 | Contestant 4 presses button | FW → iOS | Notify: EVT_STOP_EVENT, btn=4, time=3102ms |
| 10 | Contestant 2 presses button | FW → iOS | Notify: EVT_STOP_EVENT, btn=2, time=3500ms |
| 11 | Race complete (all 4 buttons) | FW → iOS | Notify: EVT_RACE_COMPLETE, winner=3, time=2345ms |
| 12 | iOS sends New Race to reset | iOS → FW | Write: [0x02, 0x00, 0x00, 0x00] |

## Example 2: Mode 2 — 1-Contestant / 4 Segments

| Step | Action | Direction | Bytes / Event |
| --- | --- | --- | --- |
| 1 | iOS connects and enables notifications | — | (same as above) |
| 2 | iOS sets Mode 2 | iOS → FW | Write: [0x06, 0x02, 0x00, 0x00] |
| 3 | iOS sends Start Race | iOS → FW | Write: [0x01, 0x00, 0x00, 0x00] |
| 4 | FW acknowledges start | FW → iOS | Notify: EVT_START_ACK (0x10) |
| 5 | Segment 1 checkpoint (Button 1) | FW → iOS | Notify: EVT_STOP_EVENT, btn=1, time=15230ms |
| 6 | Segment 2 checkpoint (Button 2) | FW → iOS | Notify: EVT_STOP_EVENT, btn=2, time=31540ms |
| 7 | Segment 3 checkpoint (Button 3) | FW → iOS | Notify: EVT_STOP_EVENT, btn=3, time=48700ms |
| 8 | Segment 4 — finish (Button 4) | FW → iOS | Notify: EVT_STOP_EVENT, btn=4, time=62100ms |
| 9 | Race complete (all 4 segments) | FW → iOS | Notify: EVT_RACE_COMPLETE, btn=0, time=62100ms |

# 8. Error Handling

## BLE Disconnection During Race

• If BLE disconnects while a race is RUNNING, the mainboard enters ERROR state but the timer keeps running.
• Buttons still work — results are still recorded locally on the mainboard.
• If the iOS app reconnects, the mainboard sends EVT_STATUS with the current elapsed time.
• The state transitions back to RUNNING if the timer is still active.
• Button 5 can still be used to manually stop the race even without BLE.

## Invalid Commands

• Sending CMD_START_RACE in a state other than CONFIGURED or ARMED → EVT_ERROR.
• Sending CMD_SET_MODE in a state other than IDLE or CONFIGURED → ignored (warning logged).
• Sending an unknown command ID → EVT_ERROR.
• Command packet shorter than 4 bytes → BLE ATT error (invalid attribute length).

## Duplicate Button Presses

• If a button that has already been recorded is pressed again during a race, the duplicate press is ignored.
• Only the first press per button is recorded and reported via EVT_STOP_EVENT.

## Reconnection Best Practices

1. After reconnecting, re-enable notifications on the Event characteristic.
2. Send CMD_GET_STATUS (0x05) to check the current state and elapsed time.
3. Read the Status characteristic to confirm the mainboard's state.
4. If the race was running, sync the UI with the received elapsed time.
5. If the race finished while disconnected, the FINISHED state will be returned.

# 9. Status Characteristic

The Status characteristic is a readable byte that maps directly to the race state:

| Value | State | Meaning |
| --- | --- | --- |
| 0x00 | DISCONNECTED | BLE not connected (you won't read this since you're connected) |
| 0x01 | IDLE | Connected, waiting for mode/configuration |
| 0x02 | CONFIGURED | Mode set, ready to start |
| 0x03 | ARMED | Armed and ready |
| 0x04 | RUNNING | Race in progress |
| 0x05 | FINISHED | Race done, results available |
| 0x06 | ERROR | Error state (e.g., previous BLE disconnect) |

# 10. Quick Reference Card

## UUIDs

Service:    00001000-7261-6365-7469-6D6572303031
Command:    00001001-7261-6365-7469-6D6572303031  (Write)
Event:      00001002-7261-6365-7469-6D6572303031  (Notify)
Status:     00001003-7261-6365-7469-6D6572303031  (Read)

## Commands (4 bytes: [cmd, mode, 0x00, 0x00])

START_RACE  = 0x01    →  Response: EVT_START_ACK (0x10)
NEW_RACE    = 0x02    →  Resets to IDLE
RESET       = 0x03    →  Resets to IDLE
PING        = 0x04    →  Response: EVT_PONG (0x14)
GET_STATUS  = 0x05    →  Response: EVT_STATUS (0x13)
SET_MODE    = 0x06    →  byte[1] = 0x01 (4-player) or 0x02 (1-player)

## Events (11 bytes: [evt, mode, btn, time(4), race_id(4)])

START_ACK     = 0x10    race started
STOP_EVENT    = 0x11    button pressed (btn=1-4, time=elapsed ms)
RACE_COMPLETE = 0x12    race finished (btn=winner, time=final ms)
STATUS        = 0x13    status response (time=current elapsed ms)
PONG          = 0x14    alive response
ERROR         = 0x1F    error occurred

## Minimal Race Flow

1. Connect to "RaceTimer"
2. Discover service 00001000-...
3. Enable notifications on Event char (00001002-...)
4. Write CMD_SET_MODE:   [0x06, 0x01, 0x00, 0x00]
5. Write CMD_START_RACE: [0x01, 0x00, 0x00, 0x00]
6. Listen for EVT_START_ACK (0x10)
7. Listen for EVT_STOP_EVENT (0x11) per button
8. Listen for EVT_RACE_COMPLETE (0x12)
9. Write CMD_NEW_RACE:   [0x02, 0x00, 0x00, 0x00]  — to reset